1. Write a program to interface LEDs with PIC18FXXXX.

// Program to interface LEDs with PIC18FXXXX

#include <xc.h>

// Configuration bits (example for PIC18F4550)

#pragma config FOSC = HS        // High-speed oscillator

#pragma config WDT = OFF        // Watchdog Timer off

#pragma config LVP = OFF        // Low-voltage programming off

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // Define oscillator frequency (8 MHz)

void main(void)

{

TRISB = 0x00;     // Set PORTB as output (LEDs connected to PORTB)

LATB = 0x00;      // Initialize all LEDs OFF

while(1)

{

LATB = 0xFF;  // Turn ON all LEDs

__delay_ms(500);

LATB = 0x00;  // Turn OFF all LEDs

__delay_ms(500);

}

}

2. Write a program to blink LEDs alternately using PIC18FXXXX.

// Program: Blink LEDs alternately using PIC18FXXXX

#include <xc.h>

// Configuration Bits (for PIC18F4550)

#pragma config FOSC = HS        // High-speed crystal oscillator

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming off

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // 8 MHz oscillator frequency

void main(void)

{

TRISB = 0x00;   // Set PORTB as output

LATB = 0x00;    // Initialize LEDs OFF

while(1)

{

LATB = 0xAA;   // 0b10101010 → LEDs ON at even positions

__delay_ms(500);

LATB = 0x55;   // 0b01010101 → LEDs ON at odd positions

__delay_ms(500);

}

}

3.Write a program to interface a button, LED, relay, and buzzer with PIC18FXXXX.

o (a) When Button 1 is pressed, the relay turns ON and LEDs chase in one

direction.

o (b) When Button 2 is pressed, the relay turns OFF and LEDs chase in the

opposite direction.

#include <xc.h>

// Configuration bits for PIC18F4550

#pragma config FOSC = HS        // High-speed oscillator

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming disabled

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // 8 MHz crystal oscillator

void delay_ms(unsigned int d)

{

while(d--)

__delay_ms(10);         // Software delay (adjust as needed)

}

void main(void)

{

signed char i;

unsigned char key = 0;

ADCON1 = 0x0F;      // Configure all pins as digital

TRISB = 0x00;       // LEDs → Output

LATB = 0x00;

// Buttons

TRISAbits.TRISA2 = 1;   // Button 1 input

TRISAbits.TRISA3 = 1;   // Button 2 input

// Relay & Buzzer outputs

TRISAbits.TRISA4 = 0;

TRISAbits.TRISA5 = 0;

LATAbits.LATA4 = 0;     // Relay OFF

LATAbits.LATA5 = 0;     // Buzzer OFF

while(1)

{

// Read buttons

if(PORTAbits.RA2 == 0) key = 1;       // Button 1 pressed

else if(PORTAbits.RA3 == 0) key = 0;  // Button 2 pressed

if(key == 1)

{

LATAbits.LATA4 = 1;   // Relay ON

LATAbits.LATA5 = 1;   // Buzzer ON

for(i = 0; i < 8; i++)   // Forward LED chase

{

LATB = (1 << i);

delay_ms(20);

LATB = 0x00;

delay_ms(20);

}

}

else

{

LATAbits.LATA4 = 0;   // Relay OFF

LATAbits.LATA5 = 0;   // Buzzer OFF

for(i = 7; i >= 0; i--)  // Reverse LED chase

{

LATB = (1 << i);

delay_ms(20);

LATB = 0x00;

delay_ms(20);

if(i == 0) break;   // Prevent underflow

}

}

}

}

4. Write a program to interface an LCD with PIC18FXXXX and display your name on the second line.

#include <xc.h>

// Configuration bits for PIC18F4550

#pragma config FOSC = HS        // High-speed oscillator

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming disabled

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // 8 MHz crystal oscillator

// Define LCD pins

#define LCD_RS LATAbits.LATA0

#define LCD_EN LATAbits.LATA1

#define LCDPORT LATB

// Send command to LCD

void lcd_cmd(unsigned char cmd)

{

LCD_RS = 0;          // Command mode

LCDPORT = cmd;

LCD_EN = 1;

__delay_ms(2);

LCD_EN = 0;

__delay_ms(2);

}

// Send data to LCD

void lcd_data(unsigned char data)

{

LCD_RS = 1;          // Data mode

LCDPORT = data;

LCD_EN = 1;

__delay_ms(2);

LCD_EN = 0;

__delay_ms(2);

}

// Initialize LCD

void lcd_init(void)

{

lcd_cmd(0x38);   // 8-bit mode, 2-line, 5x7 font

lcd_cmd(0x0C);   // Display ON, cursor OFF

lcd_cmd(0x06);   // Increment cursor

lcd_cmd(0x01);   // Clear display

__delay_ms(2);

}

void main(void)

{

ADCON1 = 0x0F;       // All pins as digital

TRISB = 0x00;        // PORTB as output

TRISA = 0x00;        // PORTA as output

LATB = 0x00;

LATA = 0x00;

lcd_init();           // Initialize LCD

lcd_cmd(0xC0);        // Move cursor to 2nd line, 1st position

// Display name on second line

lcd_data('H');

lcd_data('E');

lcd_data('M');

lcd_data('L');

lcd_data('A');

lcd_data('T');

lcd_data('A');

while(1);             // Hold display

}

5. Write a program to interface an LCD with PIC18FXXXX and display your name on the first line.

#include <xc.h>

// Configuration bits for PIC18F4550

#pragma config FOSC = HS        // High-speed crystal oscillator

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming disabled

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // 8 MHz oscillator

// LCD pin definitions

#define LCD_RS LATAbits.LATA0

#define LCD_EN LATAbits.LATA1

#define LCDPORT LATB

// Send command to LCD

void lcd_cmd(unsigned char cmd)

{

LCD_RS = 0;          // Command mode

LCDPORT = cmd;

LCD_EN = 1;

__delay_ms(2);

LCD_EN = 0;

__delay_ms(2);

}

// Send data (character) to LCD

void lcd_data(unsigned char data)

{

LCD_RS = 1;          // Data mode

LCDPORT = data;

LCD_EN = 1;

__delay_ms(2);

LCD_EN = 0;

__delay_ms(2);

}

// Initialize LCD

void lcd_init(void)

{

lcd_cmd(0x38);   // 8-bit, 2-line, 5x7 font

lcd_cmd(0x0C);   // Display ON, cursor OFF

lcd_cmd(0x06);   // Increment cursor

lcd_cmd(0x01);   // Clear display

__delay_ms(2);

}

void main(void)

{

ADCON1 = 0x0F;       // Configure all pins as digital

TRISB = 0x00;        // PORTB as output (data)

TRISA = 0x00;        // PORTA as output (control)

LATB = 0x00;

LATA = 0x00;

lcd_init();           // Initialize LCD

lcd_cmd(0x80);        // Set cursor to first line, first position

// Display your name on the first line

lcd_data('H');

lcd_data('E');

lcd_data('M');

lcd_data('L');

lcd_data('A');

lcd_data('T');

lcd_data('A');

while(1);             // Hold display forever

}

6. Write a well-documented program to interface a 4×4 keypad with PIC18FXXXX.

#include <xc.h>

// CONFIGURATION BITS (adjust as per hardware)

#pragma config FOSC = HS        // High-speed oscillator

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming disabled

#pragma config PBADEN = OFF     // PORTB<4:0> as digital I/O

#define _XTAL_FREQ 8000000      // 8 MHz crystal frequency

// LCD Pin Definitions

#define LCD_EN  LATAbits.LATA1

#define LCD_RS  LATAbits.LATA0

#define LCDPORT LATB

// Lookup table for 4×4 keypad

const unsigned char KeyLookupTbl[] = {

'1','4','7','.',

'2','5','8','0',

'3','6','9','#',

'A','B','C','D'

};

// Simple delay for LCD timing

void lcd_delay(unsigned int time)

{

unsigned int i, j;

for(i = 0; i < time; i++)

for(j = 0; j < 50; j++);

}

// Send command to LCD

void SendInstruction(unsigned char command)

{

LCD_RS = 0;              // Instruction mode

LCDPORT = command;

LCD_EN = 1;

lcd_delay(5);

LCD_EN = 0;

lcd_delay(5);

}

// Send data (character) to LCD

void SendData(unsigned char lcddata)

{

LCD_RS = 1;              // Data mode

LCDPORT = lcddata;

LCD_EN = 1;

lcd_delay(5);

LCD_EN = 0;

lcd_delay(5);

}

// LCD Initialization

void InitLCD(void)

{

ADCON1 = 0x0F;           // Configure all pins as digital

TRISB = 0x00;            // PORTB → output (LCD data)

TRISAbits.TRISA0 = 0;    // RS pin output

TRISAbits.TRISA1 = 0;    // EN pin output

SendInstruction(0x38);   // 8-bit, 2-line, 5x7 display

SendInstruction(0x0C);   // Display ON, cursor OFF

SendInstruction(0x06);   // Increment cursor

SendInstruction(0x01);   // Clear display

SendInstruction(0x80);   // Set cursor to first line

}

// Function to read a pressed key

unsigned char ReadKey(void)

{

unsigned char row, val, i, j, key = 0;

while(1)  // Wait for a key press

{

LATD = 0xFF;   // All columns HIGH

for(i = 0x01; i < 0x10; i = i << 1)  // Scan each column

{

LATD = ~i;                        // Drive one column LOW

lcd_delay(2);

row = PORTD >> 4;                 // Read rows (upper nibble)

for(j = 0x01; j < 0x10; j = j << 1)  // Check which row is LOW

{

if((row & j) == 0)            // Key pressed

{

val = KeyLookupTbl[key];   // Return corresponding ASCII

return val;

}

else

{

key++;                     // Increment key index

}

}

}

}

}

// Main Program

void main(void)

{

unsigned char Key;

unsigned char *msg = "Key Pressed = ";

TRISD = 0xF0;   // RD7–RD4 → Input (Rows), RD3–RD0 → Output (Columns)

LATD  = 0xFF;   // Initialize PORTD

InitLCD();      // Initialize LCD

SendInstruction(0x80);   // Cursor to 1st line

while(*msg)

SendData(*msg++);    // Display "Key Pressed ="

while(1)

{

Key = ReadKey();         // Wait for key press

SendInstruction(0xC0);   // Move cursor to 2nd line

SendData(Key);           // Display pressed key

lcd_delay(100);          // Debounce delay

}

}

7. Generate a square wave using a timer with interrupt (use LED bank).

#include <PIC18F4550.h>

// CONFIGURATION BITS

#pragma config FOSC = HSPLL_HS, PLLDIV = 5, CPUDIV = OSC1_PLL2

#pragma config WDT = OFF, LVP = OFF, PBADEN = OFF

unsigned char led_state = 0;

void timer0_init(void)

{

T0CON = 0b00000111;     // 16-bit mode, prescaler 1:256

TMR0H = 0xB3;           // Preload for ~1s delay

TMR0L = 0x0D;

}

void interrupt_init(void)

{

RCONbits.IPEN = 1;          // Enable priority

INTCONbits.GIE = 1;         // Enable global interrupts

INTCONbits.PEIE = 1;        // Enable peripheral interrupts

INTCONbits.TMR0IE = 1;      // Enable Timer0 interrupt

INTCONbits.TMR0IF = 0;      // Clear interrupt flag

INTCON2bits.TMR0IP = 0;     // Low priority

}

void __interrupt(low_priority) LowISR(void)

{

if (INTCONbits.TMR0IF == 1)

{

TMR0ON = 0;               // Stop timer

INTCONbits.TMR0IF = 0;    // Clear flag

// Reload timer for consistent period

TMR0H = 0xB3;

TMR0L = 0x0D;

// Toggle LEDs (square wave output)

if (led_state == 0)

{

LATB = 0xFF;

led_state = 1;

}

else

{

LATB = 0x00;

led_state = 0;

}

TMR0ON = 1;              // Restart timer

}

}

void main(void)

{

TRISB = 0x00;   // Configure PORTB as output

LATB = 0x00;    // Start with LEDs off

interrupt_init();

timer0_init();

TMR0ON = 1;     // Start Timer0

while (1);      // Infinite loop

}

8. Write an embedded C program to display numbers from 0 to 9 on a seven-segment display.

#include <pic18f4550.h>

// CONFIGURATION BITS

#pragma config FOSC = HS        // High-speed crystal

#pragma config WDT = OFF        // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage programming disabled

#define _XTAL_FREQ 20000000     // 20 MHz Crystal Frequency

// Lookup table for 0–9 (Common Anode)

const unsigned char segData[10] = {

0xC0,   // 0

0xF9,   // 1

0xA4,   // 2

0xB0,   // 3

0x99,   // 4

0x92,   // 5

0x82,   // 6

0xF8,   // 7

0x80,   // 8

0x90    // 9

};

// Simple delay

void delay_ms(unsigned int ms)

{

while(ms--)

__delay_ms(1);

}

void main(void)

{

unsigned char i;

TRISB = 0x00;   // PORTB → Output (segments)

LATB  = 0xFF;   // All segments OFF initially

while(1)

{

for(i = 0; i < 10; i++)    // Loop from 0–9

{

LATB = segData[i];     // Send pattern to 7-segment

delay_ms(1000);        // 1 second delay

}

}

}

9.Write a program to interface a button, LED, relay, and buzzer with PIC18FXXXX.

o (a) When Button 1 is pressed, the relay turns ON and LEDs chase in one

direction.

o (b) When Button 2 is pressed, the relay turns OFF and LEDs chase in the

opposite direction.

It is exactly the same as your PROBLEM STATEMENT 03 in your uploaded file

10. Interface a temperature sensor with Arduino.

int sensorPin = A0;    // LM35 output connected to analog pin A0

float temperature;     // Variable to store temperature value

void setup() {

Serial.begin(9600);  // Initialize Serial Monitor

}

void loop() {

int sensorValue = analogRead(sensorPin);  // Read analog value (0–1023)

// Convert ADC value to voltage

float voltage = sensorValue * (5.0 / 1023.0);

// LM35 outputs 10mV per °C, so temperature = voltage * 100

temperature = voltage * 100;

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

delay(1000);  // Update every 1 second

}

11. Interface a fingerprint sensor with Arduino.

#include <Adafruit_Fingerprint.h>

#include <SoftwareSerial.h>

// Define RX and TX pins

SoftwareSerial mySerial(2, 3);  // (RX, TX)

Adafruit_Fingerprint finger = Adafruit_Fingerprint(&mySerial);

void setup() {

Serial.begin(9600);           // Serial Monitor

while (!Serial);              // Wait for serial

delay(100);

Serial.println("Fingerprint Sensor Test");

finger.begin(57600);

if (finger.verifyPassword()) {

Serial.println("Sensor detected successfully!");

} else {

Serial.println("Fingerprint sensor not found :(");

while (1);  // Halt if not detected

}

Serial.println("Place finger on sensor to scan...");

}

void loop() {

getFingerprintID();   // Continuously check for fingerprint

delay(1000);

}

int getFingerprintID() {

uint8_t p = finger.getImage();

if (p == FINGERPRINT_NOFINGER) {

Serial.println("No finger detected.");

return -1;

} else if (p != FINGERPRINT_OK) {

Serial.println("Error capturing image.");

return -1;

}

p = finger.image2Tz();

if (p != FINGERPRINT_OK) {

Serial.println("Image conversion failed.");

return -1;

}

p = finger.fingerFastSearch();

if (p == FINGERPRINT_OK) {

Serial.print("✅ Match found! ID: ");

Serial.println(finger.fingerID);

Serial.print("Confidence: ");

Serial.println(finger.confidence);

} else {

Serial.println("❌ No match found.");

}

return finger.fingerID;

}

12. Interface a servo motor with Arduino.

#include <Servo.h>      // Include Servo library

Servo myservo;          // Create servo object

int pos = 0;            // Variable to store servo position

void setup() {

myservo.attach(9);    // Attach servo signal pin to D9

Serial.begin(9600);

Serial.println("Servo Motor Initialized");

}

void loop() {

// Rotate servo from 0° to 180°

for (pos = 0; pos <= 180; pos += 10) {

myservo.write(pos);           // Move servo to 'pos'

Serial.print("Angle: ");

Serial.println(pos);

delay(500);                   // Wait for servo to reach the position

}

// Rotate servo back from 180° to 0°

for (pos = 180; pos >= 0; pos -= 10) {

myservo.write(pos);

Serial.print("Angle: ");

Serial.println(pos);

delay(500);

}

}

13. Interface a humidity sensor with Arduino.

#include "DHT.h"

#define DHTPIN 7        // Data pin connected to D7

#define DHTTYPE DHT11   // Type of sensor (DHT11 or DHT22)

DHT dht(DHTPIN, DHTTYPE);  // Create DHT object

void setup() {

Serial.begin(9600);

Serial.println("DHT11 Humidity and Temperature Sensor");

dht.begin();  // Initialize sensor

}

void loop() {

delay(2000);  // Wait 2 seconds between readings

float humidity = dht.readHumidity();      // Read humidity

float temperature = dht.readTemperature(); // Read temperature (°C)

// Check for failed readings

if (isnan(humidity) || isnan(temperature)) {

Serial.println("❌ Failed to read from DHT sensor!");

return;

}

Serial.print("🌡 Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("💧 Humidity: ");

Serial.print(humidity);

Serial.println(" %");

Serial.println("------------------------");

}